# Master's Thesis: Energy Efficiency in 5G Multi-Operator Indoor Systems Utilizing Fiber-to-the-Antenna (FTTA)

DocID: SE2508-008 
Revision: A1
Class: Confidential, 
Date: 2025-08-29

# Revision history

| Date | Rev | Description | Sign |
| --- | --- | --- | --- |
| 2025-08-29 | A1 | Master's Thesis: Energy Efficiency in 5G Multi-Operator Indoor Systems Utilizing Fiber-to-the-Antenna (FTTA) | PATJI |

# Master's Thesis: Energy Efficiency in 5G Multi-Operator Indoor Systems Utilizing Fiber-to-the-Antenna (FTTA)

This thesis will investigate the impact of diverse 5G traffic scenarios on energy consumption within a multi-operator indoor system, specifically comparing Fiber-to-the-Antenna (FTTA) deployments against traditional passive Distributed Antenna Systems (DAS) using coaxial networks.

## Background and Motivation

The rapidly evolving 5G landscape, characterized by demands for higher data rates and increased capacity, presents significant challenges for energy efficiency, particularly in indoor environments. Traditional passive DAS, while prevalent, often struggle to meet the escalating uplink data rate requirements of 5G, a critical bottleneck highlighted in recent industry reports (e.g., Ericsson Mobility Reports). FTTA, by extending optical fiber directly to the antenna, offers substantially higher uplink data rates, addressing this limitation. However, this enhanced performance often comes with increased energy consumption compared to coaxial solutions, a concern explicitly identified by telecommunication operators such as Orange France. This research is directly motivated by these industry challenges and the recognized need for sustainable 5G infrastructure.

### Research Objectives

This thesis aims to:

- Quantify the energy consumption of FTTA-based multi-operator indoor systems under various realistic 5G traffic scenarios (e.g., high-density user environments, diverse service types).

- Compare the energy efficiency of FTTA deployments against traditional coaxial DAS for indoor 5G coverage.

- Analyze and evaluate the effectiveness of proposed power-saving features integrated into next-generation indoor system platforms.

- Develop theoretical models to predict energy consumption and the impact of power-saving features.

- Conduct empirical measurements on laboratory equipment to validate theoretical findings and assess real-world performance.

- Provide actionable insights for optimizing energy consumption in 5G indoor systems, with both technical and commercial implications.

### Methodology

The research will employ a mixed-methods approach, combining theoretical analysis with practical experimentation:

- Theoretical Analysis: This will involve developing mathematical models and simulations to understand the relationship between 5G traffic patterns, system configurations (FTTA vs. DAS), and energy consumption. The analysis will also incorporate the proposed power-saving features of our platform. Looking ahead, the analysis will address which features can or should be included in 6G to make active DAS an energy-efficient option for indoor 6G coverage.

A conventional RF-fed active DAS needs to infer possible power-saving modes from information transmitted over the air interface. The ORAN open fronthaul standard provides a higher level interface between the baseband and the radio unit, the analysis will look into what additional information is available that can further optimise energy efficiency in a DAS connected via such an interface.

- Laboratory Measurements: Extensive measurements will be conducted on our proprietary equipment in a controlled lab environment. These experiments will simulate various 5G traffic scenarios to gather empirical data on energy consumption with and without the implemented power-saving features.

- Data Analysis and Evaluation: The collected data will be rigorously analyzed to evaluate the effectiveness of FTTA and the proposed power-saving mechanisms, identifying key energy consumption drivers and potential areas for optimization.

### Expected Outcomes and Contributions

This thesis is expected to yield:

- A comprehensive understanding of the energy footprint of FTTA in 5G multi-operator indoor systems.

- Validated models and empirical data demonstrating the impact and benefits of specific power-saving techniques.

- Recommendations for energy-efficient design and deployment strategies for future 5G indoor infrastructure.

- Findings that will contribute to both technical advancements in telecommunications equipment design and commercial strategies for energy cost reduction in 5G networks.

Collaboration opportunities with external partners, such as Tele2, for accessing real-world traffic data and potentially conducting joint measurements, may be explored as the project progresses to further enhance the research's practical relevance.